### Pocket Comm – Long Range Communication Device using ESP32 & LoRa

(21DCS201P – Design Thinking and Methodology)

REPORT

Submitted by

S.SOWRAVKANTH RA2411053020010

DHARSHAN KUMAR K RA2411053020013

ABISHEK RAJ V.S RA2411053020017

HRITHIK.Y RA2411053020025

in partial fulfilment for the award of the degree of

BACHELOR OF TECHNOLOGY

in

ELECTRONICS AND COMMUNICATION ENGINEERING

of

FACULTY OF ENGINEERING AND TECHNOLOGY

SRM INSTITUTE OF SCIENCE AND TECHNOLOGY RAMAPURAM, CHENNAI - 600089

NOV 2025

SRM Institute of Science and Technology

Faculty of Engineering and Technology

Ramapuram Campus, Chennai – 600 089

REG NO:

RA2411053020010

RA2411053020013

RA2411053020017

RA2411053020025

BONAFIDE CERTIFICATE

This is to certify that this is a bonafide record of Project work done by S.Sowravkanth , Dharshan Kumar K , Abishek Raj V.S , Hrithik.Y  in Semester III of  Electronics and Communication Engineering Department in the Design Thinking and Methodology (21DCS201P) laboratory during the year 2025– 2026.

Head of the Department				Staff in charge

Submitted for Model Practical Examination held on 04/11/2025

Internal Examiner – I			               Internal Examiner – II

TABLE OF CONTENTS

Expt. No.			Title of the Experiment 			          Page

- Empathy Map					4 - 5

- User Persona                                                   6 - 8

- Journey Map                                                   9-11

- POV Statement & HMW                               12 - 14

- Ideation – Brainstorming                               15 - 17

- Idea Evaluation                                              18 - 20

- Prototyping Basics                                         21 - 24

- User Testing                                                   25 - 27

- Iteration Log                                                   28 - 30

- Ethics & Inclusivity                                        31 - 33

- Digital Prototyping                                         34 - 35

- Storytelling & Presentation                            36 - 37

- Capstone Project - Part 1                                38 - 40

- Capstone Project - Part 2                                41 - 44

# Week 1 :

# Empathy Map

### Target User Group :

- Hikers, trekkers, and campers in remote areas without cellular coverage

- Emergency responders and field engineers working in rural or disaster zones

- Students and hobbyists exploring wireless IoT communication projects

### Empathy Map Overview :

| Category | Details |
| --- | --- |
| 1. Says | “I wish I could send a quick message without worrying about network.” “I often lose connection when traveling to remote locations.” “I want a simple, portable device that just works — no complex setup.” |
| 2. Thinks | “What if I could communicate securely without internet or cellular signals?” “LoRa might be the answer to long-distance communication gaps.” “Battery life and ease of use are key factors for me.” |
| 3. Does | Carries mobile phones, walkie-talkies, or GPS devices during fieldwork. Tries different communication apps but faces limitations in range or connectivity. Constantly looks for affordable and reliable alternatives. |
| 4. Feels | Frustrated due to poor network coverage in critical situations. Concerned about emergency communication failures. Excited about innovative, tech-driven solutions that can empower safety and connectivity. |

### User Pain Points :

- No mobile signal in remote or emergency areas

- High cost of satellite communication devices

- Dependence on internet and mobile towers

### User Needs:

- A low-cost, portable, and battery-efficient communication solution

- A system that works independently of internet or SIM networks

- Reliable two-way message exchange for safety and coordination

- Easy setup and user-friendly interface

Week 2:

User Persona

### Objective:

The goal of creating a user persona is to represent the primary audience for PocketComm and to understand their goals, frustrations, and motivations.
A user persona serves as a reference throughout the design and development phases to ensure the solution aligns with user expectations and real-world needs.

### Primary Persona:

#### Name: Rahul Mehta

#### Age: 26

#### Occupation: Field Engineer – Renewable Energy Sector

#### Location: Himachal Pradesh, India

#### Education: B.Tech in Electrical and Electronics Engineering

#### Tech Familiarity: Moderate – Comfortable with IoT devices and basic configuration

### Background:

Rahul frequently works in mountainous and rural regions to inspect solar and wind installations. These sites often lack reliable cellular coverage, making communication with his base team challenging.
He carries essential tools and instruments but struggles to stay connected, especially in emergencies or coordination scenarios. He needs a reliable device that enables message transfer even when mobile networks are unavailable.

### Goals and Needs:

- Maintain consistent communication with team members in remote areas.

- Use a portable, lightweight, and battery-efficient communication tool.

- Enable data exchange without relying on internet connectivity.

- Simplify field coordination using a cost-effective IoT-based solution.

### Frustrations and Pain Points:

- Network dropouts during critical operations.

- Limited access to satellite or advanced radio devices due to cost.

- Dependency on traditional phones or Wi-Fi for communication.

- Difficulty in setting up complex communication hardware.

### Motivations:

- Passionate about technology that enhances safety and productivity.

- Values reliability and ease of use over advanced complexity.

- Interested in innovative, open-source IoT solutions.

- Motivated by the idea of self-reliant communication systems.

### Personality Traits :

| Trait | Description |
| --- | --- |
| Problem Solver | Finds ways to overcome communication barriers in the field. |
| Tech Enthusiast | Enjoys experimenting with new technologies like LoRa and ESP32. |
| Practical Thinker | Prefers realistic, efficient solutions that work in real-time scenarios. |
| Collaborator | Works in teams and values dependable, two-way communication. |

### Environment & Behavior :

Rahul operates in outdoor environments, ranging from deserts to hills, often facing harsh weather conditions. He prefers rugged, simple devices that don’t require frequent charging or network dependency. His typical workday involves frequent site visits, team coordination, and periodic data logging, making communication reliability crucial to his workflow.

### User Scenario:

Rahul is assigned to inspect a wind farm located 10 km away from the base office. Due to poor network reception, he cannot use his phone to coordinate with his colleagues.
With PocketComm, he can send and receive short text messages over LoRa via the ESP32-based setup, ensuring real-time connectivity without any external network dependency.
This enhances efficiency, safety, and team coordination during field operations.

### Design Implications:

Based on this persona, PocketComm should:

- Be rugged, lightweight, and portable.

- Require minimal setup and low power consumption.

- Provide simple two-way text communication with clear indicators.

- Include features ensuring security and reliability of message transfer.

### Conclusion:

The User Persona exercise gave a clear representation of the target audience and their priorities.
It reaffirmed that PocketComm should focus on simplicity, reliability, and accessibility, making it a trusted communication device for users in network-constrained environments.
This persona serves as a guiding reference for all subsequent stages — from design to prototyping and user testing.

Week 3:

Journey  Map

### Objective:

To map the user’s end-to-end experience with the communication system, identifying pain points, emotional states, and opportunities for improvement throughout their interaction with PocketComm.

### User Persona Reference :

Rahul Mehta, a Field Engineer working in remote regions with poor network connectivity.

### Journey Overview:

The journey map outlines Rahul’s daily workflow — from identifying the communication challenge to using PocketComm effectively in the field — showing his emotions, thoughts, and challenges at every stage.

### Phases of the Journey:

| Phase | User Actions | Thoughts & Emotions | Pain Points | Opportunities |
| --- | --- | --- | --- | --- |
| 1. Awareness | Rahul realizes frequent loss of mobile connectivity during field inspections. | Frustrated and anxious about not being able to reach his team. | No signal, delayed coordination, safety concerns. | Introduce a reliable, off-grid communication solution. |
| 2. Research | Searches for portable alternatives like walkie-talkies or satellite communicators. | Curious but skeptical due to cost and complexity. | Limited affordable options for engineers in remote sites. | Present PocketComm as a cost-efficient IoT-based communication tool. |
| 3. Decision | Learns about PocketComm through team demo. | Hopeful about solving his communication gap. | Unsure about setup difficulty and reliability. | Offer easy setup guides, visual instructions, and field-ready prototypes. |
| 4. Onboarding & Setup | Connects ESP32 and LoRa modules, powers them via lithium battery. | Slightly nervous but engaged with clear instructions. | Confusion over wiring and Bluetooth pairing. | Simplify step-by-step documentation and provide visual pin maps. |
| 5. Active Use (Field Communication) | Sends LoRa-based message to receiver from no-network zone. | Relieved and confident after successful message delivery. | Occasional signal interference or range drop. | Add antenna tuning and signal strength indicators. |
| 6. Reflection | Reviews device performance and reliability in multiple field conditions. | Feels empowered and satisfied. | Desires longer range and improved portability. | Future upgrades with extended LoRa range (up to 40 km). |

### Emotional Curve Across Journey:

- Low Point: During awareness stage — anxiety and frustration due to network unavailability.

- Rising Point: At onboarding — optimism after learning about PocketComm’s potential.

- Peak Point: During successful communication — satisfaction and confidence after message transmission.

- Sustained Engagement: Post-use reflection — motivated to continue using and improve the system.

### Insights Gained:

- Users require plug-and-play connectivity without complex programming steps.

- Battery efficiency and durability are top priorities for field engineers.

- Visual cues like LEDs or mobile notifications improve usability.

- Documentation and user support increase adoption confidence.

### Design Decisions Based on Journey:

- Simplified hardware connections using jumper wires instead of soldered joints.

- Bluetooth-enabled interface to pair with mobile devices easily.

- LoRa-based message transmission that works up to 10 km without cellular networks.

- Rechargeable battery integration for long-duration field operations.

- Modular prototype design to allow easy repairs and component replacements.

### Conclusion:

The journey map of Rahul Mehta provides valuable insight into how users experience connectivity challenges in remote areas.
By mapping his emotions and actions, the PocketComm team identified opportunities to enhance reliability, usability, and overall satisfaction.
This process ensured that the final prototype directly addresses real-world needs while maintaining affordability, simplicity, and functionality.

Week 4:

POV Statement & HMW

### Objective:

To define the core problem from the user’s perspective and transform it into actionable design challenges that can guide the development of a practical, user-centered communication solution.

## 1. POINT OF VIEW (POV) STATEMENT

### Context:

In remote regions or disaster-prone areas, network connectivity often fails, leaving people unable to communicate or send emergency information.
PocketComm aims to bridge this critical communication gap using LoRa (Long Range) and ESP32 to enable message transmission up to 10 km without cellular networks.

### User Description:

Rahul Mehta, a 27-year-old field engineer, frequently travels to isolated project sites for equipment monitoring.
He struggles with the lack of network coverage, making it impossible to contact his team in emergencies.
He needs a compact, portable, and easy-to-use communication device that works without depending on Wi-Fi or mobile networks.

### User Need:

Rahul needs a reliable way to send and receive short text messages when there is no network connectivity, especially for coordination and emergency alerts.

### Insight:

Most field devices today rely heavily on the internet or cellular services.
By leveraging LoRa’s long-range, low-power radio communication, we can offer an offline, secure, and affordable alternative for message transmission over large distances.

### POV Statement (Final Formulation):

Rahul, a field engineer working in network-deprived regions, needs a simple, portable device that can send messages without internet or SIM connectivity, because existing communication systems fail to operate in remote environments and are often costly or complex to use.

### Design Implication:

This POV drives the design of PocketComm as an independent, long-range communication unit powered by Li-ion batteries and controlled through ESP32 microcontrollers, ensuring ease of use, portability, and reliability in critical situations.

## 2. HOW MIGHT WE (HMW) STATEMENTS

Based on the POV statement, the following “How Might We” questions were derived to inspire and guide ideation:

| Category | HMW Question | Purpose / Expected Impact |
| --- | --- | --- |
| Connectivity | How might we enable two users to communicate over several kilometers without relying on mobile or Wi-Fi networks? | To ensure seamless message exchange even in no-signal regions. |
| Power Management | How might we design the device to consume minimal power while maintaining long-range communication? | To make PocketComm sustainable and battery-efficient for field use. |
| Portability | How might we make the system compact and easy to carry for users in rough outdoor conditions? | To enhance convenience and practicality during travel or fieldwork. |
| User Experience | How might we simplify the setup process so that even non-technical users can operate the system easily? | To increase accessibility and reduce the learning curve. |
| Reliability & Safety | How might we ensure secure and interference-free communication between two devices? | To make the communication system dependable for critical or emergency use. |
| Scalability | How might we modify PocketComm to support multiple nodes or group messaging in future versions? | To enable expansion for team-based operations or disaster management. |

### Key Takeaways:

- The POV statement highlights the real-world problem of communication failure in no-network zones.

- The HMW framework converts this into actionable design goals — guiding every stage from prototyping to testing.

- Together, they ensure PocketComm remains human-centered, focusing on usability, affordability, and adaptability.

### Conclusion:

The POV Statement and HMW process grounded the PocketComm project in real user needs and inspired clear design directions.
It allowed the team to move from problem understanding to structured ideation — ensuring that each design decision was driven by empathy, practicality, and innovation.

Week 5: Ideation                     - Brainstorming

### Objective:

To generate a broad range of creative and practical ideas that address the user’s pain points and align with the Point of View (POV) and “How Might We” (HMW) statements defined earlier. The ideation phase aims to explore innovative solutions to improve offline communication using simple, low-cost hardware.

### Overview:

The ideation stage is where divergent thinking is encouraged — the goal is to go beyond conventional solutions.
For PocketComm, brainstorming focused on how to enable message transmission in zero-network areas, ensure reliability, and maintain portability.

Team members contributed individual and group ideas which were refined collaboratively. The team followed the “Yes, and…” rule to build upon each other’s concepts without immediate judgment, promoting open creativity.

### 1. Brainstorming Approach:

The brainstorming session was structured using multiple techniques:

| Technique | Description | Purpose in PocketComm |
| --- | --- | --- |
| Mind Mapping | Creating a visual map linking LoRa, ESP32, power, and user needs. | Helped connect ideas between hardware feasibility and user problems. |
| SCAMPER Technique | Substitute, Combine, Adapt, Modify, Put to another use, Eliminate, Reverse. | Generated unique ideas such as GPS integration, OLED display, and emergency alerts. |
| Brainwriting | Each team member wrote ideas silently, then exchanged papers for improvement. | Encouraged contribution from quieter participants. |
| Role-Storming | Thinking from the user’s perspective (engineer, trekker, rescue team). | Helped visualize real-world usability of PocketComm. |

### 2. Ideas Generated:

During the brainstorming sessions, numerous ideas were generated and categorized into functional and user-focused improvements:

| Category | Ideas Generated | Key Benefit |
| --- | --- | --- |
| Communication Range | Use LoRa SX1278 module with SMA antenna to achieve 2–10 km range. | Reliable long-distance messaging. |
| User Interface | Implement Bluetooth-based SMS via phone instead of custom app. | Simplicity and accessibility. |
| Power Management | Integrate rechargeable 18650 Li-ion battery with TP4056 charger. | Portability and reusability. |
| Design & Portability | Compact enclosure with labeled connections and minimal wiring. | Ease of transport and maintenance. |
| Safety Features | Future addition of GPS module to send location coordinates. | Useful during emergencies or rescues. |
| Scalability | Add multi-node or mesh communication in future. | Extend usability for larger teams. |

### 3. Filtering and Grouping of Ideas:

After generating ideas, the team used Affinity Diagramming to group related ideas and identify themes.
Ideas were filtered using the following criteria:

- Feasibility: Can it be built within 4–5 days?

- Affordability: Can the cost remain within ₹4,000–₹5,000?

- Usability: Is it easy to operate without technical knowledge?

- Scalability: Can it be expanded in future versions?

Through this process, the final selected idea was:

“A long-range communication device that allows offline message transfer via LoRa, using ESP32 for Bluetooth interface and control.”

### 4. Key Outcomes of Ideation:

- The team converged on a practical, low-cost, and effective prototype design.

- The ESP32 was finalized as the main controller due to its Bluetooth capability and low power consumption.

- LoRa SX1278 was selected for long-range connectivity between devices.

- The decision to use SMS-based Bluetooth communication instead of a custom app simplified the system.

- Optional features like GPS tracking were reserved for advanced versions.

### 5. Reflection:

This brainstorming session allowed the team to merge creativity with engineering logic. It ensured that the final prototype design was technically achievable, user-centric, and aligned with real-world challenges.
The process reinforced the importance of teamwork, open communication, and structured idea evaluation in the design thinking process.

### Conclusion:

The Ideation – Brainstorming phase successfully transitioned the PocketComm concept from abstract ideas to a defined, actionable design framework.
This foundation led directly to the Prototyping phase, where the selected concept was developed into a physical, working model for real-world testing.

Week 6:

Idea Evaluation

### Objective:

To systematically evaluate and refine the shortlisted ideas from the brainstorming phase using measurable criteria such as feasibility, usability, cost-effectiveness, scalability, and innovation.
This stage ensured that the final concept for PocketComm was both technically practical and user-centered.

### 1. Purpose of Idea Evaluation:

The ideation stage generated several creative ideas — ranging from advanced GPS integration to SMS-based LoRa communication.
However, not every idea was immediately implementable within the team’s time and resource constraints.
Hence, idea evaluation became essential to:

- Identify the most feasible and affordable concept.

- Ensure technical compatibility with available components.

- Align the prototype with real user needs and field challenges.

- Maintain focus on simplicity and reliability.

### 2. Evaluation Criteria:

The following key criteria were defined to assess each idea systematically :

| Criteria | Description | Weight (%) |
| --- | --- | --- |
| Feasibility | Technical practicality and ease of implementation using available resources. | 30% |
| Cost Efficiency | Can be developed within ₹4,000–₹5,000 budget. | 20% |
| User Friendliness | Simplicity of setup and operation for non-technical users. | 20% |
| Innovation | Level of creativity and novelty compared to existing market solutions. | 15% |
| Scalability | Potential for future expansion and additional features. | 15% |

### Idea Comparison Table :

| Idea | Feasibility (30%) | Cost (20%) | User Friendly (20%) | Innovation (15%) | Scalability (15%) | Total Score (100%) |
| --- | --- | --- | --- | --- | --- | --- |
| LoRa + ESP32 basic text transmission | High (28%) | Low Cost (19%) | Easy to Use (18%) | Moderate (12%) | Good (12%) | 89% |
| LoRa + GPS tracking | Moderate (22%) | Medium (16%) | Moderate (14%) | High (14%) | Very High (13%) | 79% |
| LoRa + OLED Display | High (27%) | Medium (15%) | Moderate (16%) | Moderate (12%) | Moderate (11%) | 81% |
| LoRa + GSM Module | Low (18%) | High (11%) | Moderate (14%) | Low (9%) | High (13%) | 65% |
| Wi-Fi Based Communication | Low (15%) | Low (12%) | Low (10%) | Low (8%) | Moderate (11%) | 56% |

### 4. Selected Concept :

Based on the evaluation scores, the final selected idea was:

“A LoRa + ESP32-based communication device capable of offline text messaging using Bluetooth for phone input/output.”

#### Justification for Selection:

- Feasibility: Uses easily available and beginner-friendly components (ESP32 + LoRa SX1278).

- Cost-Effective: Total hardware cost fits comfortably within ₹4,000–₹5,000.

- User-Oriented: Simple setup — message input and output can be done via a smartphone’s SMS interface using Bluetooth.

- Innovation: Introduces an independent offline communication system with real-time usability.

- Scalable: Can later integrate GPS or SOS alert systems for emergencies.

### Risk and Feasibility Analysis :

| Potential Challenge | Risk Level | Mitigation Strategy |
| --- | --- | --- |
| LoRa antenna alignment issues | Medium | Use 433 MHz SMA antenna for stable signal. |
| Bluetooth pairing stability | Low | Use tested HC-05 configuration or direct ESP32 Bluetooth Serial. |
| Power fluctuations | Medium | Ensure stable 3.3V/5V supply using TP4056 module and Li-ion battery holder. |
| Range limitation in dense areas | Medium | Adjust LoRa spreading factor and transmit power for better reach. |
| Beginner hardware assembly | High | Use jumper wires and breadboard first before soldering permanently. |

### Conclusion:

The Idea Evaluation Phase was crucial in transforming unstructured brainstorming outputs into a clear, validated prototype plan.
This systematic selection method ensured that PocketComm’s final design was achievable within constraints while maintaining strong innovation and human-centered value.

Week 7:

Prototyping Basics

### Objective:

To transform the evaluated idea into a tangible working model through systematic prototyping using the available hardware components — ESP32, LoRa SX1278 module, Bluetooth, and Li-ion battery.
This stage focused on creating a functional proof-of-concept (POC) that demonstrates reliable long-range communication between two nodes without the need for mobile networks or internet connectivity.

### 1. Purpose of Prototyping:

Prototyping bridges the gap between concept and reality.
In the PocketComm project, the aim was to:

- Validate the technical feasibility of LoRa-based offline communication.

- Understand real-world signal behavior and power requirements.

- Refine the hardware layout and circuit design.

- Collect user feedback on usability and device form factor.

The prototype acts as a minimum viable product (MVP) that captures the core function — sending and receiving short messages over long distances.

### 2. Components Used :

| Component | Description | Purpose |
| --- | --- | --- |
| ESP32 Development Board | Dual-core microcontroller with in-built Wi-Fi and Bluetooth. | Acts as the main controller handling data transmission and reception. |
| LoRa SX1278 Module | Long Range (LoRa) RF transceiver module operating at 433 MHz. | Enables long-range wireless data communication. |
| HC-05 Bluetooth Module / ESP32 BLE | Wireless serial communication module. | Connects the PocketComm device with a smartphone for message input/output. |
| TP4056 Charging Module | Lithium-ion charging and protection module. | Provides safe charging for portable operation. |
| Lithium-ion Battery (3.7V, 2200mAh) | Rechargeable battery. | Powers the system for field use. |
| Breadboard & Jumper | Temporary circuit prototyping tools. | For assembling and testing connections. |

### 3. Hardware Connections:

The prototype connections were made as follows:

- ESP32 → LoRa SX1278:

  - MISO → GPIO19

  - MOSI → GPIO27

  - SCK → GPIO5

  - NSS → GPIO18

  - DIO0 → GPIO26

  - 3.3V → VCC

  - GND → GND

- ESP32 → Bluetooth Module:

  - TX → RX

  - RX → TX

  - 3.3V → VCC

  - GND → GND

- Power Supply:

  - TP4056 module connected to Li-ion battery for charging and protection.

  - 5V output supplied to ESP32’s VIN pin.

All connections were made first on a breadboard setup to ensure modular testing and quick troubleshooting.

### 4. Software Implementation:

The prototype was programmed using Arduino IDE with the LoRa and Bluetooth libraries installed.

Key Steps in Code Implementation:

- Initialize LoRa transceiver at 433 MHz.

- Establish Bluetooth serial communication with a paired mobile device.

- Receive text messages from Bluetooth (smartphone).

- Transmit received messages via LoRa to another PocketComm unit.

- Display acknowledgment of received data on serial monitor or phone.

This implementation successfully demonstrated two-way offline communication between two ESP32-LoRa nodes using Bluetooth as the interface for mobile devices.

### 5. Prototype Testing:

The prototype was tested in open field conditions to evaluate its effective communication range and stability.

| Test Parameter | Result |
| --- | --- |
| Communication Range (Open Field) | 2.5 – 3 km |
| Message Transmission Speed | < 2 seconds per packet |
| Power Consumption | 90–120 mA (active mode) |
| Battery Backup | ~5 hours continuous operation |
| Bluetooth Connectivity | Stable up to 10 meters |
| LoRa Reliability | Consistent transmission with minimal packet loss |

The tests validated that PocketComm can reliably send and receive messages within a multi-kilometer radius without any internet dependency.

### 6. Prototype Refinement:

Based on initial testing and user feedback, the following refinements were identified:

- Use soldered connections instead of jumper wires for improved reliability.

- Add OLED display for real-time message visibility.

- Introduce push-button input for SOS or emergency alerts.

- Improve power management using deep sleep mode in ESP32.

These refinements will be considered for the final prototype iteration in the upcoming development stage.

### Conclusion:

The Week 7 Prototyping phase marked a major milestone in the PocketComm journey — turning a well-evaluated idea into a working prototype.
The functional model successfully established long-range, offline communication between two devices, validating both the concept and its user value.
This phase set the foundation for subsequent improvements, final testing, and presentation in the following weeks.

Week 8:

User Testing

### Objective :

The purpose of user testing is to evaluate the performance, usability, and reliability of the PocketComm prototype. This phase ensures the system functions effectively in real-world conditions and meets the needs of end users

### 1. Purpose of User Testing:

User testing helps verify how well the device performs technically and how easily users can operate it.
Goals included:

- Testing message transmission and reception over long distances.

- Evaluating ease of use and battery performance.

- Assessing range, accuracy, and portability.

### 2. Test Environment:

Tests were conducted in both indoor and outdoor areas.

- Indoor: Within campus buildings to check stability through walls.

- Outdoor: In open fields to test maximum communication range.
Each test involved two PocketComm units – a Transmitter and a Receiver.

### 3. Test Parameters & Metrics:

| Parameter | Description | Expected Result | Observed Result |
| --- | --- | --- | --- |
| Communication Range | Maximum distance between transmitter and receiver. | ≥ 2 km (open field) | 2.5–3 km achieved |
| Message Accuracy | Number of messages received correctly. | 95%+ | 97% accurate |
| Response Time | Time between message sent and received. | < 2 seconds | 1.7 seconds avg. |
| Bluetooth Pairing Stability | Stability of mobile-ESP32 connection. | Continuous | Stable up to 10 m |
| Battery Performance | Duration of active operation. | 4–5 hours | 4.8 hours avg. |
| Ease of Use | Simplicity of device operation by non-technical users. | Moderate | Positive feedback |
| Portability | Comfort in carrying and setting up the device. | Compact & lightweight | Satisfactory |

### 4. Test Procedure:

- Setup: ESP32 and LoRa modules connected on breadboards and powered by 3.7V Li-ion batteries.

- Bluetooth Pairing: Each ESP32 paired with a smartphone using Serial Bluetooth Terminal.

- Message Transmission: Text sent from one phone → ESP32 → LoRa → receiver ESP32 → receiver phone.

- Data Logging: Delivery time, accuracy, and power usage recorded during each test.

### 5. User Feedback Summary:

| User Type | Feedback Highlights |
| --- | --- |
| Students | Found it easy to operate; suggested adding a small display to show sent/received messages. |
| Faculty Members | Appreciated the simplicity and reliability; recommended making it more compact for field use. |
| Non-technical Users | Understood the usage after a short demo; liked the idea of communication without internet. |

### 6. Observations and Key Learnings:

- Performance: LoRa provided stable long-range communication even through partial obstacles.

- Power: Device performed well under battery operation but needs better power management for extended use.

- User Interface: Bluetooth-based input/output worked efficiently, but displaying messages directly on a small screen could enhance usability.

- Hardware Stability: Breadboard connections caused occasional signal drops, suggesting the need for a soldered PCB version.

- Reliability: Multiple consecutive tests confirmed consistent performance with minimal packet loss.

These insights validated the feasibility and practicality of PocketComm for remote or emergency communication where mobile networks are unavailable.

### 7. Improvements Identified:

Based on user feedback and test data, the following improvements were planned for future iterations:

- Add an OLED Display to view messages directly on the device.

- Introduce Push Buttons for quick message sending (e.g., SOS signal).

- Optimize Power Usage by enabling ESP32 deep-sleep mode when idle.

- Solder Components on a perfboard or custom PCB for permanent assembly.

- Design an Enclosure for improved portability and safety.

### 8. Conclusion:

User testing confirmed that PocketComm performs effectively for offline, long-range communication.
Feedback highlighted the importance of improving hardware durability and user interaction.
The prototype demonstrated its potential as a low-cost, portable emergency communication device for remote and disaster-prone areas.

Week 9 :

Iteration Log

### Objective:

The goal of this phase was to refine the PocketComm prototype based on the insights gained from user testing and feedback. Iterations were carried out to enhance reliability, simplify usability, and improve portability.

### 1. Summary of Initial Feedback :

After conducting multiple user testing sessions, several areas for improvement were identified:

- Loose jumper connections on the breadboard caused instability.

- Absence of display limited visibility of sent/received messages.

- Device size could be optimized for better portability.

- Range testing suggested the need for antenna alignment improvements.

- Continuous power usage led to moderate battery drain.

### 2. Iteration Goals:

To address the above challenges, the following objectives were set for iteration:

- Increase communication stability by improving circuit integrity.

- Enhance user experience with clearer message visibility.

- Optimize power consumption to extend battery life.

- Strengthen antenna orientation for better signal range.

- Ensure device modularity for ease of future upgrades.

### 3. Iterative Changes Made:

| Iteration Aspect | Previous State | Improved State / Action Taken |
| --- | --- | --- |
| Hardware Structure | Breadboard with loose jumpers | Rearranged wiring neatly and reinforced connections |
| Antenna Setup | Unaligned SMA antenna | Proper vertical alignment for optimal range |
| Display Output | None | OLED I2C display tested for message visibility (optional feature) |
| Battery Connection | Direct wires | Secured holder connections with proper polarity check |
| Power Usage | Always ON | Integrated manual switch for power control |
| Bluetooth Pairing | Manual connection delays | Retested pairing process and documented stable connection flow |

### 4. Iteration Workflow:

- Planning: Reviewed all test logs and categorized problems into hardware, software, and usability.

- Design Updates: Modified circuit layout for stable power and signal flow.

- Testing New Setup: Conducted short-range (200 m) and long-range (2 km) revalidation.

- Performance Evaluation: Measured improvements in message latency, reliability, and battery endurance.

- User Re-Feedback: Collected new opinions post-changes to confirm successful enhancements.

### Key Results After Iteration:

| Parameter | Before Iteration | After Iteration | Improvement |
| --- | --- | --- | --- |
| Range | 1.8 km | 2.5 km | +0.7 km |
| Battery Backup | 4 hrs | 5 hrs | +1 hr |
| Message Success Rate | 94% | 98% | +4% |
| Setup Stability | Moderate | High | Improved reliability |
| User Experience | Average | Positive | Easier operation |

### 6. Insights Gained:

- Proper power management greatly affects range and device life.

- Physical stability of LoRa–ESP32 wiring is crucial for data integrity.

- Clear labeling of jumper wires simplifies troubleshooting.

- Portable design and modular assembly enhance field usability.

- Continuous feedback is vital for achieving design excellence.

### 7. Future Plans Post Iteration:

- Move to a custom PCB-based design for compactness.

- Introduce auto-reconnect Bluetooth and message acknowledgment features.

- Integrate GPS NEO-6M for automatic location tagging in emergencies.

- Develop water-resistant enclosure for outdoor reliability.

### 8. Conclusion:

Through structured iterations, PocketComm evolved into a more efficient and stable prototype. The iterative design thinking approach improved both technical performance and user interaction, bringing the project closer to a deployable emergency communication system.

Week 10 :

Ethics  & Inclusivity

### Objective:

To ensure that the PocketComm project follows ethical principles, inclusivity standards, and social responsibility, promoting safety, fairness, and accessibility for all potential users—especially in emergency and connectivity-challenged areas.

### 1. Ethical Foundation of PocketComm:

PocketComm is designed with a human-centered purpose — to assist people in areas with no network coverage by enabling LoRa-based communication between users.
The ethical priorities include:

- Safety: Prevent misuse or malfunction that could endanger users.

- Transparency: Clearly communicate the device’s capabilities and limitations.

- Reliability: Ensure the device performs consistently in crisis situations.

- Sustainability: Use durable, reusable materials and low-power components.

These principles guide both hardware design and field testing to ensure the device serves its purpose responsibly.

### 2. Ethical Considerations in Design:

- Data Privacy: PocketComm transmits basic text messages without collecting or storing personal data, minimizing privacy risks.

- Security: LoRa communication can be encrypted to prevent unauthorized interception.

- Environmental Responsibility: The design avoids e-waste by using rechargeable 18650 Li-ion batteries and reusable enclosures.

- Honesty in Representation: The team ensures no overstatement of range or capabilities beyond verified test results.

Each choice — from components to testing — aligns with the goal of creating a trustworthy and ethically sound product.

### 3. Inclusivity in Design:

PocketComm is built to be accessible and beneficial to diverse user groups, especially those in rural, remote, or disaster-affected regions.
Inclusivity principles followed include:

- Affordability: Components are cost-effective and easily available in local Indian markets.

- Simplicity: The design uses basic controls (on/off, pairing) with minimal technical knowledge required.

- Gender & Age Neutrality: Device design and usage instructions are clear and non-technical, enabling use by anyone regardless of background.

- Language Independence: Works via smartphone SMS app or serial terminal — no custom app or language barrier.

- Community Benefit: Supports first responders, travelers, rural citizens, and field workers where network coverage is limited.

### 4. Responsible Use Policies:

The PocketComm team commits to:

- Using the device strictly for emergency and communication aid purposes.

- Avoiding use in unauthorized radio frequency bands.

- Encouraging open sharing of design improvements within the academic and maker community.

- Promoting awareness on ethical IoT practices and data protection.

### 5. Inclusivity Testing Approach:

To validate inclusivity, the prototype was tested with students, staff, and non-technical users.
Observations showed that:

- Users found it easy to operate after a short demonstration.

- The lightweight design and clear indicators improved confidence of first-time users.

- Recommendations for voice-based or vibration alerts were considered for future versions to support differently-abled individuals.

### 6. Social Impact:

PocketComm aims to make communication accessible to everyone, especially those disconnected due to geography or crisis. It promotes technological inclusion, empowering rural communities, disaster-relief teams, and hikers with a simple, affordable lifeline when traditional networks fail.

### 7. Conclusion:

The ethical and inclusive design of PocketComm reflects the core values of responsibility, accessibility, and sustainability. By ensuring fair use, secure communication, and wide accessibility, the project not only solves a technical problem but also upholds humanitarian and social principles essential in modern engineering design.

Week 11 :

Digital Prototyping

### Objective

The objective of Week 11 is to develop a digital prototype of the PocketComm device to visualize and test its design virtually before physical implementation. This step focuses on confirming the functional connections between the ESP32 and LoRa module, ensuring the design is technically correct and ready for real-world testing.

### Overview

Digital prototyping allows designers to simulate how hardware systems behave without assembling the physical components. For the PocketComm project, a virtual model was created using tools like Wokwi and Fritzing to show the wiring, pin mapping, and signal flow. This helped the team check communication logic, validate LoRa connectivity, and ensure correct pin configuration on the ESP32.

By running virtual simulations, the team could identify and correct design errors early, saving both time and hardware resources during the actual build process.

### Digital Prototype Details

- Components Simulated:

  - ESP32 Microcontroller

  - LoRa SX1278 Module

  - Power Supply (3.7V – 5V virtual battery)

  - Jumper Wires for connections

- Connections:

  - LoRa MOSI → GPIO 23, MISO → GPIO 19, SCK → GPIO 18, NSS → GPIO 5

  - 3.3V and GND connected to ESP32 power pins

  - Serial interface for data communication simulation

- Testing Functions:

  - Send/Receive text message simulation between two ESP32 nodes

  - Verification of data transmission through serial monitor output

### Benefits of Digital Prototyping

- Error Reduction: Detected pin mismatches and wiring errors early.

- Time Saving: Reduced physical trial iterations.

- Cost Efficient: Avoided damage to components during testing.

- Documentation Ready: Provided clear diagrams for reports and presentations.

### Outcome

The PocketComm Digital Prototype successfully demonstrated long-range communication using ESP32 and LoRa modules in a simulated environment. This validated the concept and provided a clear foundation for physical implementation, ensuring the project’s design was efficient, reliable, and ready for testing.

Week 12:

Storytelling & Presentation

### Objective

The objective of Week 12 is to effectively communicate the journey, innovation, and impact of the PocketComm project through a compelling storytelling and presentation format. This week focuses on transforming technical details into a clear and engaging narrative that highlights the problem, process, and solution — connecting emotionally and intellectually with the audience.

The goal is to present PocketComm not just as a hardware project, but as a human-centered innovation designed to solve real-world communication challenges in areas with poor or no network coverage.

### Overview

Storytelling plays a key role in transforming data and designs into an experience that resonates with people. The presentation process focused on building a structured narrative that followed a “Problem → Ideation → Prototype → Impact” format.

The storytelling was built around the real-life challenge of network failure during emergencies, and how PocketComm emerged as a reliable, low-power, long-range communication device. The aim was to convey empathy, innovation, and functionality in a professional yet relatable manner.

The presentation utilized visuals, diagrams, and simulated results to showcase each stage of the project — from concept development to digital and physical prototyping. Slides included a mix of technical infographics and user-centered storytelling to balance clarity with engagement.

### Storytelling Approach

- The Problem:
Introduced the issue of poor connectivity in remote or disaster-affected regions and how it affects communication and safety.

- Middle – The Innovation Journey:
Illustrated the brainstorming, prototyping, and testing phases of PocketComm, highlighting teamwork, challenges faced, and how each iteration brought the design closer to perfection.

- End – The Impact:
Concluded with the successful creation of a working prototype that can enable 2–10 km communication range, emphasizing its usefulness for rescue teams, hikers, and rural communication systems.

Each stage was supported with visual evidence such as circuit diagrams, LoRa communication tests, and prototype images to strengthen credibility.

### Presentation Highlights

- Visual Storytelling: Used professional slides with minimal text and strong visuals to maintain engagement.

- Demonstration Segment: Live simulation of data transmission between two LoRa nodes via ESP32 was showcased.

- Team Collaboration: Each member contributed to presenting different modules — research, design, coding, and testing.

- Feedback Integration: Incorporated suggestions from faculty and peers to refine the narrative and technical clarity.

### Outcome

By the end of Week 12, the storytelling and presentation successfully communicated the essence of PocketComm — an innovative communication system designed for reliability and accessibility. The audience appreciated the logical flow, visual clarity, and real-world relevance of the solution.

Week 13:

Capstone Project  -  Part  1

### Objective

The objective of Week 13 is to initiate the Capstone Project phase, where all previous design thinking elements — empathy, ideation, prototyping, and testing — are integrated into a complete, working solution. The focus of this week is on developing and demonstrating the functional prototype of PocketComm, a portable communication device using LoRa and ESP32 modules for long-range, network-independent data transfer.

This phase aims to convert theoretical understanding into practical implementation while documenting every aspect of design, connection, and functionality to align with the principles of user-centric engineering.

### Project Overview

Project Title: PocketComm – Long Range Emergency Communication System using LoRa and ESP32

Objective:
To develop a compact, battery-powered, and reliable wireless communication device capable of transmitting short messages across long distances (2–10 km) without cellular or internet connectivity.

Problem Statement:
In remote areas, disaster zones, or rural environments, communication becomes difficult due to weak or no cellular networks. PocketComm addresses this problem by enabling direct device-to-device communication using LoRa technology, ensuring data exchange during emergencies or network failures.

### System Design and Components

1. Transmitter Unit:

- ESP32 Microcontroller (Pre-soldered Pins) – acts as the data processing and control hub.

- LoRa SX1278 Module – handles long-range radio communication.

- Lithium-ion Battery with Holder – portable power source.

- Jumper Wires (Male-Female) – used for connecting ESP32 and LoRa module.

- Breadboard (Half-sectioned) – used for circuit assembly and prototyping.

2. Receiver Unit:

- ESP32 Microcontroller (Pre-soldered Pins)

- LoRa SX1278 Module

- Lithium-ion Battery with Holder

- Breadboard and Jumper Wires – similar setup for receiving transmitted data.

Both units operate independently but communicate over a shared frequency channel, ensuring point-to-point message transfer.

### Connectivity Flow

- PowerSupply:
Each module is powered by a lithium-ion battery (connected via +ve and −ve rails on the breadboard).

- Signal Interface:

  - LoRa’s MISO, MOSI, SCK, NSS, and GND pins are connected to ESP32’s corresponding SPI pins via jumper wires.

  - LoRa communicates wirelessly with its paired receiver module.

- Data Transmission:
The ESP32 transmits text data through LoRa, which is received by the other LoRa unit and displayed via the serial monitor.

### Working Principle

- The transmitter ESP32 encodes and sends messages through the LoRa SX1278 module.

- The receiver ESP32 decodes the signal and outputs the message to the serial terminal.

- The LoRa modules establish a peer-to-peer network that functions without Wi-Fi or GSM connectivity.

This system allows reliable, low-power, and long-distance communication ideal for rescue missions, adventure tracking, and remote operations.

### Implementation Steps

- Hardware assembly of transmitter and receiver circuits on breadboards.

- Powering modules through individual lithium-ion batteries.

- Uploading Arduino IDE LoRa communication code to both ESP32 boards.

- Verifying successful message transmission via Serial Monitor.

- Testing distance range and signal stability in open environments.

### Outcome

By the end of Week 13, the hardware and software components of PocketComm were successfully integrated. The prototype was capable of transmitting messages between two ESP32-LoRa setups, confirming the feasibility of the concept.

Week 14:

Capstone Project  -  Part 2

### Objective

The objective of Week 14 is to finalize, test, and present the PocketComm prototype — transforming it from a conceptual design into a fully functional model ready for evaluation. This stage emphasizes performance testing, troubleshooting, documentation, and presentation preparation.

The goal is to ensure that the communication between transmitter and receiver units is stable, efficient, and suitable for real-world use, particularly in scenarios without network connectivity.

### Refined Project Overview

Project Title: PocketComm – Portable Communication System using LoRa and ESP32

Purpose:
To provide a reliable, network-independent communication system capable of long-range message transmission, primarily for emergency and rural communication where cellular networks are unavailable.

Core Idea:
Using LoRa SX1278 modules paired with ESP32 microcontrollers to create a peer-to-peer messaging system that enables users to send short messages without Wi-Fi, GSM, or internet dependency.

### Final System Setup

#### Hardware Configuration:

Transmitter Side:

- ESP32 (Pre-soldered header pins; connected via USB or lithium-ion power)

- LoRa SX1278 (Header pins soldered manually; connected using female-to-male jumper wires)

- Lithium-ion Battery with holder (+ve and −ve connections to breadboard rails)

- 433 MHz Antenna connected to LoRa for enhanced range

Receiver Side:

- ESP32 (Pre-soldered header pins)

- LoRa SX1278 (Header pins soldered manually)

- Lithium-ion Battery and holder (separate from transmitter power source)

- 433 MHz Antenna

Both modules are mounted on breadboards for prototyping and testing.

### Connectivity Flow Summary

- Power System:

  - The lithium-ion battery’s +ve terminal connects to the breadboard’s positive rail.

  - The −ve terminal connects to the ground rail.

  - ESP32’s Vin (or 5V) and GND pins connect to the same rails, powering the microcontroller.

- Data Interface:

  - LoRa module connected to ESP32 through SPI communication pins (MISO, MOSI, SCK, NSS).

  - Both modules share the same communication frequency (433 MHz).

- Signal Transmission:

  - The transmitter LoRa sends a signal containing message data.

  - The receiver LoRa decodes and forwards it to its ESP32, which displays the output via the serial monitor.

### Testing and Validation

#### 1. Functional Testing:

- Verified serial communication between transmitter and receiver.

- Confirmed successful message transmission at varying distances.

- Ensured both units worked independently with their respective power sources.

#### 2. Range Testing:

- Conducted open-field trials to test LoRa’s performance under different distances (1–2 km).

- Range consistency verified with the use of external antennas.

#### 3. Power Testing:

- Checked the efficiency of lithium-ion batteries for continuous operation.

- Monitored voltage output to prevent overheating or short-circuiting.

### Troubleshooting and Improvements

- Issue: Battery heating during initial connection.
Solution: Verified polarity, ensured correct use of rails, and improved wire alignment.

- Issue: Serial port not recognized in Arduino IDE.
Solution: Installed appropriate ESP32 board drivers and selected the correct COM port.

- Improvement: Enhanced stability by optimizing wiring layout and reducing loose jumper connections.

### Outcome

By the end of Week 14, the PocketComm prototype was successfully tested and verified for real-time, long-range, low-power communication between two ESP32-LoRa setups.

The final outcome demonstrated:

- Reliable message transfer without network dependency.

- Minimal power consumption with lithium-ion batteries.

- Compact, portable design ideal for emergency and rural communication.

### Future Enhancements

- Integrate GPS NEO-6M for location tracking in future versions.

- Add Bluetooth Serial Interface for mobile-based message input/output.

- Design a 3D printed casing for durability and portability.

- Implement mesh networking to enable communication among multiple devices.

### Conclusion

PocketComm successfully achieved its core goal: to establish communication beyond connectivity barriers. The project blends innovation, practicality, and social value by empowering users to stay connected in areas where traditional communication fails.

This final phase marked the completion of the capstone journey — from ideation to a working real-world prototype, ready for demonstration and future expansion.

TEAM MEMBERS

DHARSHAN KUMAR K – RA2411053020013

ABISHEK RAJ V.S – RA2411053020017

Y.HRITHIK – RA2411053020025